SinkHole Attack in LEACH

Software Recommended: NetSim standard v13.3, Visual Studio 2022

Project Download Link:

Follow the instructions specified in the following link to download and setup the Project in NetSim:

https://support.tetcos.com/en/support/solutions/articles/14000128666-downloading-and-setting-upnetsim-file-exchange-projects

Low – Energy Adaptive Clustering hierarchy(“LEACH”):

Leach is a MAC protocol which is integrated with clustering and a simple routing protocol in wireless sensor networks (WSN). The goal of LEACH is to lower the energy consumption required to create and maintain clusters to improve the lifetime of a wireless sensor network.

This Cross-Layer Protocol is implemented in NetSim in the MAC layer which involves ZigBee Protocol and the Network layer which involves DSR protocol. The clustering of sensors happens in the Network Layer and the cluster head selection involves interacting with the MAC layer to obtain the remaining power of the sensors.

A LEACH.c file is added to the DSR project.

For this implementation of LEACH, the number of Clusters is fixed as 4 and all the 4 clusters are equal. If the user wants to change it, then he/she must also change the static routing for the Cluster Heads and the Cluster Element array accordingly.

Figure 1: Network setup for Sinkhole attack in LEACH

Figure 2: Leach.c file in source code

To make 4 equal clusters the number of sensors must be 4,16,36,64,100. Depending on the number of sensors, the ClusterElements array must be defined. Here, it has been defined and commented on for 4,16,36,64,100 sensors. Uncomment the one you want to use.

The File contains the following functions:

fn_NetSim_LEACH_CheckDestination() // to check whether the current device is the destination or not.

fn_NetSim_LEACH_GetNextHop() // For getting the next hop device id.

fn_NetSim_LEACH_AssignClusterHead() // For electing the Cluster head based on Remaining energy.

fn_NetSim_LEACH_IdentifyCluster() // To determine the cluster to which a sensor belongs.

Sinkhole Attack on LEACH:

In this project, we are implementing a sinkhole attack on top of the LEACH project where a malicious node advertises false battery information to become a cluster head. Upon being elected as a cluster head, it attracts network traffic from all its cluster members and destroys the packets without forwarding them to the sink/base station.

Implementation:

A file malicious.c is added to the DSR project which contains the following functions:

fn_NetSim_DSR_MaliciousNode() This function is used to identify whether a current device is malicious or not in order to establish malicious behavior.

fn_NetSim_DSR_MaliciousProcessSourceRouteOption() This function is used to drop the received packets if the device is malicious, instead of forwarding the packet to the next hop.

You can set any device as malicious and you can have more than one malicious node in a scenario. Device IDs of malicious nodes can be set inside the fn_NetSim_DSR_MaliciousNode() function.

Steps:

- To open-source code open NetSim > Your Work > Source Code > Open code

In LEACH.c present inside DSR Project, the number of clusters and Size of clusters are decided, and in Malicious. c the malicious node is set You can set them to any value, and the node

Note: By default, Malicious Node is set to 4 and NUMBER OF CLUSTERS – 4, SIZE OF CLUSTERS – 16, If changed Rebuild the Solution as shown below:

Figure 3: solution Rebuild in source code

Rebuild the solution by right clicking on solution > Rebuild solution.

The Workspace_Sinkhole_in_LEACH comes with an inbuilt network scenario example, Open the scenario.

Run the simulation.

Results and discussion:

View the packet animation. You will note that the sensors directly start transmitting packets without route establishment since the routes are statically defined in LEACH. You will also note that the cluster heads keep changing dynamically in Clusters 2, 3, and 4. In cluster1, the cluster members transmit packets to the malicious node (device id 4) since it advertises false battery information to become a cluster head.

This can be observed in the Packet trace by applying filters to the Source_ID column by selecting only Sensor-1, 12, 20, 28. You will be able to see that the receiver id is sensor-4 throughout the simulation. All the nodes in Cluster1 are sending data packets to the malicious node (Sensor-4) since it is the Cluster Head

Figure 4: Packet trace file referring to malicious node reception of transmitted packets

This will have a direct impact on the Application Throughput which can be observed in the Application Metrics table present in the NetSim Simulation Results window. The throughput for applications 1, 4, 6, and 8 is zero throughputs since the source ids belong to cluster1 having a malicious node (device id 4)

Figure 5: Simulation results window